Before you begin:

- Download the user manual:
Visit www.dnrbroadcast.com, click the Support button on the homepage, and select Brochures / Manuals / Software. Download the Webstation User Manual and read it at least once.

- Safety reminder:
Turn OFF both the Webstation and other equipment before connecting them to avoid grounding issues that may damage input circuits.
Power up the equipment in the following order:

  - Turn ON the Webstation.

  - Turn ON the connected equipment.

- Maintenance tips:

  - Clean the front panel periodically.

  - Avoid smoking, eating, or drinking near the console.

### 1️. Downloading and installing required software

- Get the latest software & firmware:

  - Visit www.dnrbroadcast.com → Support → Service Information / FAQ’s.

  - Go to the Webstation product page and download all required software.

- Install the software:

  - Double-click the downloaded .exe file and follow on-screen instructions.

- Update the firmware:

  - Use the D&R Firmware Update Tool (instructions in User Manual, Chapter 1.5).

- Additional software:

  - Webstation Configuration Manager & Webstation VoIP setup is explained in User Manual, Chapters 1.3 & 1.4 (Pages 32 & 41).

### 2️. Setting up the USB modules

The Webstation has 4 stereo USB signals for your PC:

- USB-1 → Main program signal

- USB-2 → Clean feed

- USB-3 → Voice track signal

- USB-4 → VoIP signal

#### Setup steps:

✔ Run an audio application on your PC to monitor audio levels (free tool: Test Generator).
✔ The return USB signal is fixed but can be adjusted using Gain controls on the Webstation.
✔ Routing of incoming USB signals is handled automatically by Windows.
✔ The ON switches connect incoming stereo signals (from Play-out system 1-3) to Channels 3, 4, and 5.

### 3️. setting up the VoIP module

The built-in VoIP interface allows connection to online services like Skype, enabling calls from listeners to your station.

#### Setup steps:

✔ Use the dedicated VoIP USB connector (back panel) for Skype.
✔ Skype can run on:

- The same PC (using a second USB port).

- A separate PC for better performance.
✔ Download Skype: www.skype.com.
✔ Adjust Skype settings for proper integration with the Webstation.

### Additional resources

- For detailed instructions:
Download the full User Manual at www.dnrbroadcast.com by selecting Support > Brochures / Manuals / Software.

- For FAQs and troubleshooting:
Visit Support > Service Information / FAQ’s on our website.

- For support and technical questions: Go to the header of the hompage of D&R, click on the SUPPORT button and choose CONTACT SUPPORT. Fill in the support form and one of our technical engineers will contact you as soon as possible.